File S4. Recommended order of operations for segmenting a new image set in OrganoSeg2.

This file is written as an if-then protocol that seeks to capture the major categories of images segmentable by OrganoSeg2. There are three main phases: I) coarse segmentation, II) conditional fine segmentation, and III) post-segmentation processing. Certain steps in earlier phases connect with go-to statements in later phases where indicated. The protocol starts with the default settings of OrganoSeg2.

Phase I: Coarse segmentation

- Conditional preliminary steps:

  - If images have poor contrast or uneven illumination, then adjust image brightness by selecting the Brightness button in the Raw Grayscale Image subpanel and applying the Local Brighten or Flatfield corrections. To compare segmentation results of raw and adjusted images side-by-side, go to Preferences>Brightness. More in-depth image adjustments are described in the OrganoSeg2 ReadMe and the associated MATLAB documentation for available contrast-adjustment functions.

  - If organoid contrast is white on a black background (e.g., fluorescence image), then select Preferences>Additional Parameters and change BrightForegroundObj from FALSE (default) to TRUE.

  - If many organoids are touching one another or overlapping in the Z plane, then select the Split Whole Image (Watershed Transform) radio button in the Segmentation Options subpanel. For refinement, see Step 6.

  - If the image has many organoids near the image edge, then select Preferences>Additional Parameters and change ClearBorderBeforeSplit from TRUE (default) to FALSE.

  - Select Out-of-Focus Correction in the Segmentation Options subpanel unless organoid sizes are very homogeneous and fast runtimes are a priority.

- Adjust the Size Threshold in the Segmentation Options subpanel until objects on the scale of the smallest true organoids are recognized. Start with a twofold increase or decrease and refine from there.

- Adjust the Window Size in the Segmentation Options subpanel in increments of 100 until increases yield no further improvements in segmentation.

- Adjust the Intensity Threshold in the Segmentation Options subpanel in increments of 0.25 with attention paid to correct object recognition rather than the object segmentation itself, which will be refined in Step 5. Decrease the Intensity Threshold if organoids are missed and increase the Intensity Threshold if contaminants are recognized. Note: segments can be removed manually in Phase III, but they cannot be added. Err on the side of false positives when decreasing the Intensity Threshold, especially at the edges of Matrigel domes or microplate wells that are easily removed en masse during Phase III.

- If decreasing the Intensity Threshold does not improve organoid recognition and introduces oversegmentation artifacts, then select the Edge Correction radio button in the Segmentation Options subpanel and repeat Step 4.

- If the default Edge Correction remains suboptimal, then select Preferences>Edge Correction to change the edge correction method in Correction Preferences:

  - Gradient + Watershed – simple and robust default for the Edge Correction radio button.

  - Gradient Preserve Boundary – uses Edge Threshold in the Segmentation Options subpanel as a tunable parameter; largely preserves original segments

  - Gradient Only – uses Edge Threshold in the Segmentation Options subpanel as a tunable parameter; no constraints on original segments

For Step 6b and 6c, adjust Edge Threshold in the Segmentation Options subpanel in increments of 0.1–0.2. Decrease Edge Threshold if organoid edges are not fully recognized and increase it if the edge captures surrounding debris or neighboring organoids. Note: if edge corrections in Steps 5–6 are problematic, revisit Step 2 with a somewhat higher Size Threshold.

Phase II: Conditional fine segmentation

- If Out-of-Focus Correction is selected, then select Preferences>Additional Parameters and increase WindowStepSize to improve run times, stopping before segmentation performance degrades. Start at one-fifth the WindowSize specified in Step 3 and adjust from there.

- If organoid recognition remains inadequate, then select Preferences>Additional Parameters and try lowering PresegReconstrSize and evaluate whether segmentation is improved. Start at zero and increase in increments of one until organoid recognition has improved as much as possible.

- If edge correction remains inadequate, then select Preferences>Additional Parameters and try changing ReconstrForEdgeCorrect from TRUE (default) to FALSE and evaluate whether segmentation is improved.

- If segmentation borders are overly jagged, then select Preferences>Additional Parameters and increase SegmentCloseSize in increments of two until segmentation borders are sufficiently smooth.

- If organoid splitting is insufficient or excessive, then select Preferences>Additional Parameters and change WatershedBlurStdDev by decreasing in increments of two to improve sensitivity and increasing in increments of two to reduce sensitivity. Note: individual organoids may be split at different WatershedBlurStdDev values in Phase III.

Phase III: Post-segmentation processing

- To split specific individual organoids with higher sensitivity by selecting the Split Selected Spheroid radio button in the Change Classification subpanel and setting the threshold to zero or one.

- Combine oversplit organoids individually by with the Combine radio button in the Change Classification subpanel and increasing the default gap if needed to join segments.

- Adjust Contaminant Intensity and Minimum Circularity in the Segmentation Options subpanel, starting at 0.5 and increasing or decreasing independently to exclude as many non-organoid segments as possible.

- Remove leftover non-organoid segments with the Remove radio button in the Change Classification subpanel